SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL2- DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry look-ahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bit-pair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Code of Conduct:

I ________________________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Identification of Errors / Bugs (20%) | Debugging Approach & Analysis (20%) | Correctness of Debugged Solution (25%) | Optimization Techniques Applied (20%) | Testing and Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

1. Signed Integer Addition Using 2’s Complement

Problem

An 8-bit processor gives an incorrect result when adding:

+120 + +50

Step 1: Convert decimal values into 8-bit binary

+120:

120₁₀ = 01111000₂

+50:

50₁₀ = 00110010₂

Step 2: Perform binary addition

01111000   (+120)

+ 00110010   (+50)

------------

10101010

The obtained result is:

10101010₂

Since the most significant bit (MSB) is 1, the result is interpreted as a negative number in 2’s complement.

Find its decimal value:

Invert:       01010101

Add 1:        01010110

01010110₂ = 86

Therefore:

10101010₂ = −86

The processor displays −86, but the actual mathematical result is:

120 + 50 = 170

Debugging Analysis

The range of an 8-bit signed number is:

−128 to +127

The correct answer, +170, is outside this range. Therefore, signed overflow has occurred.

Both input numbers are positive, but the output is negative. This is the main indication of overflow.

Overflow Detection Rule

For signed addition:

Positive + Positive → Negative result = Overflow

Negative + Negative → Positive result = Overflow

Positive + Negative = No signed overflow

Another hardware rule is:

Overflow = Carry into MSB XOR Carry out of MSB

Solution

Add an overflow detection circuit.

Check the sign bits of both operands and the result.

Use a larger data width, such as 16-bit representation.

For 16-bit representation:

120 + 50 = 170

The result is correctly represented.

Final Conclusion

The incorrect result is caused by 8-bit signed overflow. The problem can be solved using an overflow flag or by increasing the register size.

2. Ripple Carry Adder Delay and Carry Look-Ahead Adder

Problem

A processor uses a ripple carry adder, but arithmetic operations are slow in a real-time system.

Working of a Ripple Carry Adder

In a ripple carry adder, each full adder must wait for the carry output from the previous stage.

For a 4-bit adder:

Bit 0 → Carry C1

↓

Bit 1 → Carry C2

↓

Bit 2 → Carry C3

↓

Bit 3 → Final Carry C4

The carry moves from one bit to the next like a ripple.

Debugging Analysis

The delay occurs because:

Bit 1 waits for the carry from Bit 0.

Bit 2 waits for the carry from Bit 1.

Bit 3 waits for the carry from Bit 2.

The final output is available only after all carry operations are completed.

For an n-bit ripple carry adder:

Delay ∝ n

Therefore, increasing the number of bits increases the delay.

Optimized Solution: Carry Look-Ahead Adder

A carry look-ahead adder calculates carry signals in advance.

It uses:

Generate signal

Gi = Ai × Bi

A carry is generated when both input bits are 1.

Propagate signal

Pi = Ai XOR Bi

A carry is propagated when one input bit is 1.

Carry equations

C1 = G0 + P0C0

C2 = G1 + P1G0 + P1P0C0

C3 = G2 + P2G1 + P2P1G0 + P2P1P0C0

The carry values are calculated simultaneously instead of waiting for each previous stage.

Debugging Comparison

| Feature | Ripple Carry Adder | Carry Look-Ahead Adder |
| --- | --- | --- |
| Carry calculation | Sequential | Parallel |
| Speed | Slow | Fast |
| Delay | High | Low |
| Hardware complexity | Simple | More complex |
| Real-time performance | Lower | Better |

Final Conclusion

The bottleneck is the sequential carry propagation. A carry look-ahead adder reduces delay by calculating carry signals in parallel.

3. Debugging Booth’s Multiplication Algorithm

Problem

Booth’s multiplication unit produces incorrect results when negative numbers are used.

Example

Multiply:

−5 × +3

Using 4-bit 2’s complement:

−5 = 1011

+3 = 0011

Initial registers:

A  = 0000

Q  = 0011

M  = 1011

Q−1 = 0

Booth’s algorithm checks the bit pair:

Q₀Q₋₁

Booth’s Rules

| Q₀Q₋₁ | Operation |
| --- | --- |
| 00 | No operation |
| 01 | A = A + M |
| 10 | A = A − M |
| 11 | No operation |

After every operation, perform an arithmetic right shift.

Debugging Points

1. Incorrect bit-pair checking

The circuit must check:

Q₀ and Q−1

If the bits are checked in the wrong order, addition and subtraction operations become incorrect.

2. Incorrect arithmetic shift

For signed multiplication, an arithmetic right shift must be used.

The sign bit must be copied during the shift.

Example:

Before shift: 10110000

After shift:  11011000

The leftmost 1 is preserved.

If a logical right shift is used:

Before shift: 10110000

After shift:  01011000

The sign becomes incorrect.

3. Incorrect sign extension

Negative values must be sign-extended.

For example:

4-bit −5 = 1011

8-bit −5 = 11111011

Incorrect zero extension:

00001011

This represents +11, not −5.

4. Incorrect subtraction

The operation:

A − M

must be performed using 2’s complement:

A − M = A + (2’s complement of M)

Corrections

Check Q₀Q₋₁ correctly.

Use the correct Booth operation.

Perform arithmetic right shifting.

Preserve the sign bit.

Use sign extension for negative values.

Run the algorithm for exactly n cycles.

Final Result

−5 × +3 = −15

The binary result is:

11110001₂

Therefore, the main causes of error are usually incorrect bit-pair checking, logical shifting, or improper sign extension.

4. Restoring Division and Non-Restoring Division

Problem

A system using restoring division takes more time because it repeatedly restores the partial remainder.

Working of Restoring Division

For every bit:

Shift the accumulator and quotient.

Subtract the divisor from the partial remainder.

Check whether the result is negative.

If negative:

Restore the previous remainder.

Set quotient bit to 0.

If positive:

Keep the result.

Set quotient bit to 1.

Debugging Analysis

The main inefficiency occurs when:

Partial Remainder − Divisor < 0

The algorithm performs:

R = R − D

Then it restores the value:

R = R + D

Therefore, subtraction and addition are performed unnecessarily in the same cycle.

This increases:

Execution time

Number of arithmetic operations

Power consumption

Optimized Solution: Non-Restoring Division

The non-restoring method does not immediately restore the remainder.

The next operation depends on the sign of the previous remainder.

Rules

If remainder is positive:

R = 2R − D

If remainder is negative:

R = 2R + D

The algorithm avoids the immediate restoration step.

Comparison

| Feature | Restoring Division | Non-Restoring Division |
| --- | --- | --- |
| Restoration operation | Required | Not required every cycle |
| Number of operations | More | Fewer |
| Execution time | Higher | Lower |
| Hardware efficiency | Lower | Better |
| Speed | Slower | Faster |

Final Conclusion

The unnecessary operation is the repeated restoration of the partial remainder. Non-restoring division improves performance by using the sign of the remainder to select the next addition or subtraction operation.

5. IEEE 754 Single-Precision Floating-Point Addition

Problem

A scientific application gives inaccurate results when a very small number is added to a very large number.

Example:

100000000 + 0.01

IEEE 754 Single-Precision Format

A 32-bit floating-point number contains:

| Field | Number of bits |
| --- | --- |
| Sign | 1 |
| Exponent | 8 |
| Fraction/Mantissa | 23 |

The value is represented as:

(−1)ˢ × 1.f × 2^(e−127)

Debugging Process

Step 1: Exponent alignment

Before addition, both numbers must have the same exponent.

Suppose:

Large number = 1.0 × 2²⁶

Small number = 1.34 × 2⁻⁷

The small number’s mantissa must be shifted right many times to match the large exponent.

After repeated shifting, the small value may become too small to fit in the available 23-bit fraction.

Step 2: Loss of precision

The shifted bits may be removed.

Therefore:

100000000 + 0.01 ≈ 100000000

The value 0.01 may not affect the final result.

This is called loss of significance or absorption error.

Step 3: Normalization

After addition, the result must be normalized.

A normalized value has the form:

1.xxxxx × 2ᵉ

If normalization is not performed correctly, the exponent or fraction may become incorrect.

Step 4: Rounding

IEEE 754 uses rounding rules.

Common rounding mode:

Round to nearest, ties to even

Rounding can introduce a small error because the exact result may not fit into the available fraction bits.

Solutions

1. Use double precision

Double precision uses:

| Field | Bits |
| --- | --- |
| Sign | 1 |
| Exponent | 11 |
| Fraction | 52 |

It provides much higher precision than single precision.

2. Add small values first

Instead of:

Large + Small + Small + Small

first calculate:

Small + Small + Small

Then add the result to the large number.

3. Use compensated summation

Algorithms such as Kahan summation store the small lost error and add it during later calculations.

4. Use higher-precision intermediate values

Perform calculations using double precision and convert to single precision only at the final output.

Final Conclusion

The inaccuracy is caused mainly by exponent alignment, limited mantissa size, and rounding. Double precision and compensated summation improve numerical accuracy.

Overall Summary

| Question | Main Problem | Debugging Result | Optimized Solution |
| --- | --- | --- | --- |
| 1 | Signed addition | 8-bit overflow | Overflow flag or larger bit width |
| 2 | Ripple carry adder | Sequential carry delay | Carry look-ahead adder |
| 3 | Booth multiplication | Bit-pair, shift, or sign error | Correct bit checking and arithmetic shift |
| 4 | Restoring division | Repeated restoration | Non-restoring division |
| 5 | IEEE 754 addition | Precision loss and rounding | Double precision or compensated summation |