SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | B.Vennela | Reg.No.: | 192412604 |
| Department: | Ece | Date: | 2/9/26 |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

1. Real-Time Embedded I/O Communication Mechanism

Design: Use vectored interrupts with memory-mapped I/O.

Architecture

Sensors

↓

I/O Interface

↓

Interrupt Controller

↓

Vectored Interrupt

↓

CPU

↓

Memory

Working

- Multiple sensors generate an interrupt every 1 ms.

- The interrupt controller receives and prioritizes these interrupts.

- In vectored interrupt handling, each device has a specific interrupt vector/address.

- CPU directly jumps to the required Interrupt Service Routine (ISR).

- Memory-mapped I/O is selected because I/O registers are treated as memory locations and accessed using normal load/store instructions.

- ISRs are kept short to reduce CPU overhead.

CPU utilization:

CPU Utilization=ISR Time1ms×100

For example, if ISR execution time is 100 µs:

1001000×100=10%

Thus, it satisfies the <15% CPU utilization requirement.

Conclusion: Vectored interrupts + memory-mapped I/O provide fast I/O response with low CPU utilization.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

Design: Use NVMe over PCIe with DMA.

- Architecture

CPU

|

PCIe / NVMe Controller

|

DMA Controller

|

Main Memory

|

NVMe SSD

- Working

NVMe SSD communicates with the CPU through the PCIe bus.

The NVMe controller uses DMA (Direct Memory Access) to transfer large blocks of data directly between the SSD and main memory.

CPU only initializes the DMA transfer and handles completion.

CPU does not copy every byte of data.

After completing the transfer, DMA/NVMe generates an interrupt to notify the CPU.

Throughput

The NVMe subsystem should support:

Throughput>3 GB/s

Therefore, a sufficiently fast PCIe/NVMe interface is required.

Conclusion: NVMe + PCIe + DMA provides high-speed storage transfer while minimizing CPU involvement.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Given Constraints

- 6 USB peripherals

- 2 high-speed Thunderbolt devices

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Proposed Architecture

CPU

│

┌─────────┴─────────┐

│ Interrupt Controller│

└─────────┬─────────┘

│

┌────────────┴────────────┐

│                         │

Hardware Interrupts       Software Interrupts

│                         │

┌──────┴──────┐            Background Tasks

│             │

USB Controller   Thunderbolt

│             Controller

│                  │

USB Hub          ┌────┴────┐

│             │         │

┌────┼────┐      TB-1      TB-2

│ │ │ │ │ │

U1 U2 U3 U4 U5 U6

1. USB Communication

- Connect six USB peripherals to a USB host controller.

- A USB hub can be used to connect multiple USB devices.

- USB controller manages:

  - Data transfer

  - Device identification

  - Error detection

  - Interrupt generation

- USB devices generate interrupts when:

  - Data is received

  - Data transfer is completed

  - An error occurs

2. Thunderbolt Communication

- Connect two high-speed devices through a Thunderbolt controller.

- Thunderbolt is used for high-bandwidth devices.

- Suitable for:

  - High-speed storage

  - Displays

  - External PCIe devices

- Thunderbolt controller handles high-speed data transfers.

3. Hardware Interrupts

- Generated directly by hardware devices.

- Used for time-critical events.

- Examples:

  - USB data arrival

  - Thunderbolt transfer completion

  - Device error

- Interrupt controller receives all hardware interrupts.

- CPU executes the corresponding Interrupt Service Routine (ISR).

4. Software Interrupts

- Generated by software/program instructions.

- Used for:

  - Background processing

  - Operating-system services

  - Deferred I/O processing

- Reduces the workload inside hardware ISRs.

- After the hardware ISR finishes, remaining processing can be handled using a software interrupt.

4.Design adata acquisition system with constraints  Continuous data stream at 500 MB/s

Minimal CPU intervention required

Must compare DMA transfer vs

interrupt-driven I/O

Use memory-mapped I/O for device registers

Given Constraints

- Continuous data stream = 500 MB/s

- Minimal CPU intervention

- Compare DMA and interrupt-driven I/O

- Use memory-mapped I/O (MMIO) for device registers

Proposed Architecture

DATA ACQUISITION SYSTEM

┌───────────────┐

│    Sensor     │

└───────┬───────┘

↓

┌───────────────┐

│      ADC      │

└───────┬───────┘

↓

┌───────────────┐

│ I/O Registers │

│     (MMIO)    │

└───────┬───────┘

↓

┌───────────────┐

│ DMA Controller│

└───────┬───────┘

↓

┌───────────────┐

│  Main Memory  │

│    Buffer     │

└───────┬───────┘

↓

┌───────────────┐

│      CPU      │

│ Data Processing│

└───────────────┘

1. Memory-Mapped I/O

- Device registers are assigned memory addresses.

- CPU accesses device registers using normal load/store instructions.

- Registers may include:

  - Control register

  - Status register

  - Data register

  - DMA configuration register

- MMIO provides simple and fast access to the acquisition hardware.

2. DMA-Based Data Transfer

- ADC continuously generates data at 500 MB/s.

- DMA controller transfers data directly from the acquisition device to main memory.

- CPU does not handle every data word.

- CPU only:

  - Configures DMA.

  - Specifies memory address.

  - Specifies transfer size.

  - Starts the DMA transfer.

  - Processes data after DMA completion.

ADC → DMA Controller → Main Memory

↓

CPU

3. Buffering

Use double buffering or circular buffering to support continuous data.

ADC

↓

DMA

↙   ↘

Buffer A   Buffer B

↓          ↓

CPU        DMA

Processing   Transfer

- DMA fills Buffer A.

- CPU processes Buffer B simultaneously.

- When Buffer A is full, DMA switches to Buffer B.

- This prevents data loss during continuous acquisition.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

. Interrupt Handling Mechanism

Given Constraints

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Proposed Architecture

┌───────────────┐

│ High Priority │

│    Devices   │

└───────┬───────┘

↓

┌─────────────────┐

│ Interrupt       │

│ Controller      │

└────────┬────────┘

↓

Vectored IRQ

↓

┌─────────────────┐

│      CPU        │

└────────┬────────┘

↓

High-Priority ISR

│

┌──────┴──────┐

│ Nested IRQ? │

└──────┬──────┘

↓

Higher-Priority ISR

↓

Return to Previous ISR

↓

Software Interrupt

↓

Background Tasks

1. Interrupt Priority

Interrupts are assigned different priority levels.

| Priority | Interrupt Source | Handling |
| --- | --- | --- |
| 1 | Critical device | Vectored hardware interrupt |
| 2 | High-speed I/O | Vectored hardware interrupt |
| 3 | Normal I/O | Hardware interrupt |
| 4 | Background task | Software interrupt |

- Higher-priority interrupts are serviced first.

- Lower-priority interrupts wait until the CPU becomes available.

2. Vectored Interrupts

- High-priority devices use vectored interrupts.

- Each device has a unique interrupt vector/address.

- When an interrupt occurs, the CPU directly jumps to the corresponding ISR.

- This reduces interrupt identification time.

Code of Conduct:

I ____________________________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |