SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2- PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 30% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | B.Vennela | Reg.No.: | 192412604 |
| Department: | Ece | Date: | 2/9/26 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Code of Conduct:

I ______B.vennela 9192412604)______________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |

1 Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement

DMA-Based Data Transfer Algorithm

Objective: Transfer data from an I/O device to memory while minimizing CPU involvement.

ALGORITHM DMA_Data_Transfer

Code :

START

1. CPU initializes DMA controller

2. Set source = I/O device

3. Set destination = Memory

4. Set transfer_size = required data size

5. Set transfer_mode = DEVICE_TO_MEMORY

6. Enable DMA controller

7. Enable DMA interrupt

8. WHILE data transfer is not complete

DMA controller transfers data

directly from I/O device to memory

CPU performs other tasks

IF DMA transfer is complete THEN

Generate DMA interrupt

END IF

END WHILE

9. CPU receives DMA interrupt

10. CPU checks DMA status

11. IF transfer successful THEN

Process received data

ELSE

Handle DMA error

END IF

12. Disable DMA interrupt

13. Return CPU to normal operation

END

Key point: CPU only configures DMA and handles completion/error interrupts; it does not transfer every data item.

2 Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

ALGORITHM Vectored_Interrupt_Handler

Code :

START

1. Initialize interrupt controller

2. Assign priority to each device

3. Assign interrupt vector to each device

4. Enable required interrupts

5. WHILE system is running

IF interrupt request occurs THEN

Identify interrupt source

Read interrupt priority

Select highest-priority pending interrupt

Save CPU context

Get interrupt vector

Jump to corresponding ISR

Execute ISR

Clear interrupt flag

Restore CPU context

Return from interrupt

END IF

END WHILE

END

3.Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

Objective: Select the appropriate I/O addressing method according to latency and bandwidth requirements.

ALGORITHM Select_IO_Method

Code :

START

1. Read device requirements

2. Measure/obtain:

latency_requirement

bandwidth_requirement

3. IF latency_requirement is HIGH THEN

Select MEMORY_MAPPED_IO

Use normal load/store instructions

ELSE IF bandwidth_requirement is HIGH THEN

Select MEMORY_MAPPED_IO

Use DMA for bulk data transfer

ELSE

Select I/O_MAPPED_IO

Use dedicated IN/OUT instructions

END IF

4. Configure selected I/O mechanism

5. Initialize device registers

6. Start data transfer

7. Monitor performance

8. IF required latency or bandwidth is not achieved THEN

Switch to the alternative I/O method

END IF

END

Selection

| Requirement | Selected Method |
| --- | --- |
| Very low latency | Memory-mapped I/O |
| High bandwidth | Memory-mapped I/O + DMA |
| Simple/low-speed I/O | I/O-mapped I/O |
| Large data transfer | MMIO + DMA |

4 Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

. Bus Arbitration Algorithm for PCIe, USB and Thunderbolt

Objective: Manage multiple devices requesting access to the communication bus fairly.

ALGORITHM Bus_Arbitration

Code :

START

1. Initialize PCIe, USB and Thunderbolt interfaces

2. Create request queue

3. WHILE system is running

Check all interfaces for bus requests

IF PCIe device requests bus THEN

Add PCIe request to queue

END IF

IF USB device requests bus THEN

Add USB request to queue

END IF

IF Thunderbolt device requests bus THEN

Add Thunderbolt request to queue

END IF

Assign priority to each request

Select highest-priority request

IF multiple requests have same priority THEN

Use ROUND_ROBIN arbitration

END IF

Grant bus access to selected device

Allow device to transfer data

IF transfer is complete OR time slice expires THEN

Release bus

Move request to end of queue

END IF

END WHILE

END

Arbitration Example

PCIe Request

↓

USB Request → Bus Arbiter → Selected Device → Data Transfer

↑

Thunderbolt Request

Important Features

- Priority-based arbitration handles urgent transfers.

- Round-robin arbitration provides fairness.

- Time slicing prevents one device from occupying the bus continuously.

- Waiting devices are eventually serviced, preventing starvation.

5.Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

. Hybrid I/O Communication Mechanism

Objective: Use polling for low-speed devices and DMA with interrupts for high-speed devices.

ALGORITHM Hybrid_IO_Communication

Code :

START

1. Detect all connected I/O devices

2. FOR each device

Measure/identify device data rate

IF data rate < LOW_SPEED_THRESHOLD THEN

Select POLLING mode

ELSE

Select DMA mode

Enable DMA interrupt

END IF

END FOR

3. WHILE system is running

FOR each LOW-SPEED device

Poll device status

IF data is available THEN

Read data

Store data in memory

END IF

END FOR

FOR each HIGH-SPEED device

Configure DMA

Transfer data directly:

Device → DMA → Memory

CPU performs other tasks

IF DMA transfer is complete THEN

Generate interrupt

CPU executes DMA ISR

Process received data

END IF

END FOR

END WHILE

END

Hybrid I/O Architecture

I/O Devices

|

┌────────┴────────┐

↓                 ↓

Low-Speed Devices   High-Speed Devices

↓                 ↓

Polling             DMA

↓                 ↓

CPU              Memory

↓

Interrupt

↓

CPU